Solar-Peltier Coupled Water

Purification System

Undergraduate Project

Department of Mechanical Engineering

IIT Kanpur

Project Supervisor

Dr. Sameer Khandekar

Group 18

Avinash Kumar (150169)

Atul Bimbrahw (150162)

Rahul Kumar (150546)

DESIGN CALCULATIONS

Area of duct = 10 x 13 cm2 ; Fan = 12 × 12 cm2 ; Air flow = 0.04 m3/s

Fin thickness = 2 mm ; Gap = 5 mm

No. of fins = n = 13/0.7 = 18 fins

Now,                                                            Re = ρvDµ                                 [Reynolds Number between two fins]

ρ = PRT = 1.15 kg/m3 ; v = 0.0410×0.5×10-4  = 80 m/s ; D = 4anp =  4×10×0.521 = 17 cm

Re = 1.15×80×0.171.81×10-5 = 864088

So,

Nu = f2Re-1000Pr⁡1 + 12.7f212(Pr23- 1)               [Nusselt Number for air flowing through duct]

Nu = (f/2)(Re – 1000)                 [Because Pr=1]

Now ,                                    f = 0.00128 + 0.1143 Re-13.2154   ; f = 2.9 × 10-3

Therefore,                                                 Nu = 1255.09

Now we know,                                              hL/k = Nu                            [Here, h & k are properties of air]

i.e          hL = 31.37 W/mK

Power of Peltier Module,

Q= Latent Heat of waterTime = 2250×1036×3600 = 104 W                            [which we now approximate to 100W]

Now, we set

h = 100 w/m2k,  L = 31.3 cm,  Q = 100W

Fin width = 2 mm, n = 18 fins ; Ac = 0.313 × 0.002 = 6.26×10-4 m2 ; Perimeter = 2×0.315 = 0.63 m

a = (hp/kA)1/2 = ( 100×0.63205×6.26×10-4)1/2   =  22.16

Now, we calculate Heat Flow rate

Q=(Heat flow rate through fins)+(Heat flow rate through base)

Q = 18 × hpkAc (Tb - T∞) tanh(aL) + 18×(Tb - T∞) h (0.5×0.313/100)

Q= 18 × [100×0.63×205×6.26×10-4 × tanh(22.16×0.1)× (Tb-T∞)]

+  18 × (Tb-T∞) × (100×0.5×0.313/100)

Q = (Tb-T∞)[49.98 + 2.817]

Q = 52.8 × (Tb - T∞)

Now we take into account the factor of safety,

ns × Q = 52.8 × (Tb - T∞)                                   [ns to take into account the loss in base temperature]

ns × 10052.8 = (80 - T∞)

ns × (1.89) = (80 - T∞)

take ns = 1.5

T∞ = 77.2 °C                                               [Maximum temperature attainable by air]

D

B

C

A

w2

w1

At   A

TA = 30 °C ; RHA = 40 %

At  B

TB = 77.4 °C ; RH = 4 %

At  C

Tsat = 77.2 °C ; w2 = 200

At D

T = 5 °C ; w1 = 40

w2 – w1 = 160 × 0.143  = 22.88 g/kg of dry air

density of water = 1g/cc

0.0228 L                      1 kg of air

1 L                       43.859 kg of air      [We are able to heat 43.859 kg of air in 6 hours to give 1 Ltr of water]

Taking factor of safety ns = 1.5

43.859 kg of air gives 666 ml of water in 6 hours

ORTHOGRAPHIC PROJECTIONS

All dimensions in mm

PARTS DESCRIPTION

Quantitative Specifications:

- 72 Aluminium Fins ; 320 x 50 x 2 mm3

- 4 Peltier Module ; 25W, 60 x 40 mm2

- Evaporation Slab: POP

- Plastic Casing

Parts to be fabricated:

- Aluminium Fins

- Evaporation Slab

Parts to be purchased:

- Peltier Module

- Plastic Casing

Approximate Cost:

- 4 Peltier Module : Rs.10000

- Casing : Rs.500

Approx. Total Cost= Rs. 10,000