ME352 LAB REPORT

GRASHOF CRITERION EXPERIMENT

Experiment number-2

Sub-group number-C3

Name and Roll number: -

- Puthawala Faisal 	150542

- Raghav Gupta 	150545

- Rahul Kumar 	150546

Date and Day Experiment was conducted: 04/09/17 Monday

Date of Submission of Report: 11/09/17 Monday

Mechanism 2B

1. Construct a Grashof and a non-Grashof 4R mechanism. Consider all possible cases of a Grashof linkage.

Ans: Grashof 4R Mechanism-

1) Double crank:

2) Crank-Rocker:

Observation Table:

| Input Angle | 0° | 20° | 40° | 60° | 80° | 100° | 120° | 140° | 160° | 180° | 200° | 220° | 240° | 260° | 280° | 300° | 320° | 340° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output Angle | 50° | 63° | 60° | 63° | 66° | 65° | 63° | 59° | 54° | 50° | 46° | 42° | 40° | 39° | 35° | 36° | 33° | 40° |

Plot for Crank-Rocker Grashof Mechanism

3) Double Rocker:

Non-Grashof 4R Mechanism-

Observation Table

| Input Angle | 78° | 60° | 40° | 20° | 0° | -20° | -40° | -60° | -78° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output Angle | 45° | 85° | 98° | 95° | 56° | 25° | 0° | -13° | -45° |

Plot for Non-Grashof Mechanism

2. Discuss the features of the plots that you obtain.

Ans: For Grashof crank-rocker, the rocker angle fluctuates for some range of crank angle and then decreases smoothly for crank angle range [100° - 250°] from maximum until it achieves its minimum.

For Non-Grashof, it can be seen that the limiting case will be a triangle, so the maximum and minimum angle will be same in absolute terms. Also, maximum/minimum of follower is achieved at maximum/minimum of the driver. Also, the forward and return path of the linkage will not be same.

3. List and explain difference that you see between the various Grashof and Non- Grashof linkages.

-In Grashof linkage sum of length of longest and smallest is less than sum of other two links. While in case of Non-Grashof, it is larger.

-Grashof linkage can be double crank, crank rocker, double rocker but non Grashof is always double rocker.

4. Verify that two instances of a Grashof linkages cannot be achieved by a single construction. Show that this is not the case for Non-Grashof linkage.

Ans. Using Grashof criterion, three different 4R mechanisms are possible and each requires a different construction. In non-Grashof linkage, only one mechanism, that is a double rocker mechanism is possible. So, any two instances of a Grashof linkage cannot be achieved by a single construction, while all constructions of non-Grashof linkage will give the same mechanism. This was verified by the constructions made in lab.

Mechanism 2C

1. Describe your number synthesis in detail

Ans: 	n=8

3n-2j-4 = 0

3*8-2j-4 = 0

- j = 10

n2+n3+n4 =8

2n2+3n3+4n4 = 2j = 20

Given n4 =0

n2+n3 =8

2n2+3n3 =8

- n2 = 4 and n3 = 4

2. Show the 8-bar mechanism that you built.

Ans:

3. Report any 8-bar constrained mechanism that you missed.

Ans: